First thing to remember

Local network is its own network

Xbox is its own network

Tiktok is its own network

Dota is its own network

Gateway connects networks to each other

Places have a gateway that leads you to the internet

Router – connects networks they move packets from one network to another.

The door to get out of a network for a client is the default gateways ip address

You can give a device a static ip address devices that are always on the network its just situational

N etwork

A ddress

T ranslation

Translating your public address to your private address of your device

method of mapping an IP address space into another by modifying network address information in the IP header of packets while they are in transit across a traffic routing device.

Diagram for NAT

Swapping source IP on the way out and destinatin IP on the way back in

Router cares about IP because it connects networks

Switch looks at MAC addresses since everything’s on the same network

Switches in a business enhances security separeates the departments. Some departments don’t need to know information form other departments. Network traffic is more efficient being split up instead of all connecting to a router.

Broadcast Domain a group of devices all connected together.

A ddress

R esolution

P rotocol

Resolves ip addresses to mac addresses

Needs the mac address because that’s how switches move data

Needs to know mac addresses since IP addresses are always changing

IP address is used to locate a device on the network mac address identifies the actual device

Table is stored with all the mac addresses and IP addresses checks the ARP cache

If cache is empty sends a broadcast message across every device to ask the mac address of an IP address. The computer that has the matching IP will respond back now that it knows creates an entry in the arp table.

DHCP heres my IP

ARP heres my MAC address

ARP cache makes the network more efficient so each time devices communicate it doesn’t have to send a a broadcast every time. Dynamic are not permanent they get flushed out occasionally. ARP adds security by storing the static address if someone trys to change it wont connect unless you have physical access to the device.

Routing deals with IP addresses. Internet protocol. Routers use tables.