Packet Tracer - Design and Implement a VLSM Addressing Scheme

# Addressing Table

| Device | Interface |  | IP Address |  | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- | --- |
| Police blank | G0/0 | 192.168.203.129 |  |  | 255.255.255.240 | N/A |
|  | G0/1 | 192.168.203.97 |  | blank | 255.255.255.224 | N/A |
|  | S0/0/0 | 192.168.203.145 |  | blank | 255.255.255.252 | N/A |
| Schools | G0/0 | 192.168.203.65 |  | blank | 255.255.255.224 | N/A |
|  | G0/1 | 192.168.203.1 |  | blank | 255.255.255.192 | N/A |
|  | S0/0/0 | 192.168.203.146 |  | blank | 255.255.255.252 | N/A |
| PD-1 | VLAN 1 | 192.168.203.130 |  | blank | 255.255.255.240 | 192.168.203.129 |
| PD-2 | VLAN 1 | 192.168.203.98 |  | blank | 255.255.255.224 | 192.168.203.97 |
| PS-101 blank | VLAN 1 | 192.168.203.66 |  | blank | 255.255.255.224 | 192.168.203.65 |
| PS-115 | VLAN 1 | 192.168.203.2 |  | blank | 255.255.255.192 | 192.168.203.1 |
| PD-1-11 | NIC | 192.168.203.142 |  | blank | 255.255.255.240 | 192.168.203.129 |
| PD-2-23 | NIC | 192.168.203.126 |  | blank | 255.255.255.224 | 192.168.203.97 |
| PS-101-87 blank | NIC | 192.168.203.94 |  | blank | 255.255.255.224 | 192.168.203.65 |
| PS-115-12 blank | NIC | 192.168.203.63 |  | blank | 255.255.255.192 | 192.168.203.1 |

Blank Line, No additional information

# Objectives

In this lab you will design a VLSM addressing scheme given a network address and host requirements. You will configure addressing on routers, switches, and network hosts.

- Design a VLSM IP addressing scheme given requirements.

- Configure addressing on network devices and hosts.

- Verify IP connectivity.

- Troubleshoot connectivity issues as required.

# Background / Scenario

You have been asked to design, implement, and test an addressing scheme for a customer. The customer has given you the network address that is suitable for the network, the topology, and the host requirements. You will implement and test your design.

## Packet Tracer - Design and Implement a VLSM Addressing Scheme

# Instructions

You have been given the network address                                                  by your customer. The host address requirements are:

# Requirements

Host Requirements:

| LAN |  | Number of Addresses Required |
| --- | --- | --- |
| PD-1 LAN | 8 |  |
| PD-2 LAN | 19 |  |
| PS-101 LAN | 21 |  |
| PS-115 LAN | 32 |  |

Blank Line, No additional information

## Design Requirements

- Create the addressing design. Follow guidelines provided in the curriculum regarding the order of the subnets.

- The subnets should be contiguous. There should be no unused address space between subnets.

- Provide the most efficient subnet possible for the point-to-point link between the routers.

- Assign the subnets in the order of the number of hosts from the greatest to the least.

- Document your design in a table such as the one below.

| Subnet Description | Number of Hosts Needed | Network Address/CIDR | First Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| PS-115 LAN | 32 | 192.168.203.0/26 | 192.168.203.1 | 192.168.203.63 |
| PS-101 LAN | 21 | 192.168.203.64/27 | 192.168.203.65 | 192.168.203.95 |
| PD-2 LAN | 19 | 192.168.203.96/27 | 192.168.203.97 | 192.168.203.127 |
| PD-1 LAN | 8 | 192.168.203.128/28 | 192.168.203.129 | 192.168.203.143 |
| WAN LINK | 2 | 192.168.203.144/30 | 192.168.203.145 | 192.168.203.147 |

Blank Line, No additional information

## Configuration Requirements

Note: You will configure addressing on all devices and hosts in the network.

- Assign the first usable IP addresses in the appropriate subnets to                              for the two LAN links and the WAN link.

- Assign the first usable IP addresses in the appropriate subnets to                              for the two LANs links. Assign the last usable IP address for the WAN link.

- Assign the second usable IP addresses in the appropriate subnets to the switches.

- The switch management interface should be reachable from hosts on all of the LANs.

- Assign the last usable IP addresses in the appropriate subnets to the hosts.

## Packet Tracer - Design and Implement a VLSM Addressing Scheme

If the addressing design and implementation are correct, all hosts and devices should be reachable over the network.

End of document