Packet Tracer - VLSM Design and Implementation Practice Topology

You will receive one of three possible topologies.

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Building1 | G0/0 | 10.11.48.97 | 255.255.255.240 | N/A |
|  | G0/1 | 10.11.48.65 | 255.255.255.224 | N/A |
|  | S0/0/0 | 10.11.48.121 | 255.255.255.252 | N/A |
| Building2 | G0/0 | 10.11.48.113 | 255.255.255.248 | N/A |
|  | G0/1 | 10.11.48.1 | 255.255.255.192 | N/A |
|  | S0/0/0 | 10.11.48.122 | 255.255.255.252 | N/A |
| ASW1 | VLAN 1 | 10.11.48.98 | 255.255.255.240 | 10.11.48.97 |
| ASW2 | VLAN 1 | 10.11.48.66 | 255.255.255.224 | 10.11.48.65 |
| ASW3 | VLAN 1 | 10.11.48.114 | 255.255.255.248 | 10.11.48.113 |
| ASW4 | VLAN 1 | 10.11.48.2 | 255.255.255.192 | 10.11.48.1 |
| Host-A | NIC | 10.11.48.110 | 255.255.255.240 | 10.11.48.97 |
| Host-B | NIC | 10.11.48.94 | 255.255.255.224 | 10.11.48.65 |
| Host-C | NIC | 10.11.48.118 | 255.255.255.248 | 10.11.48.113 |
| Host-D | NIC | 10.11.48.62 | 255.255.255.192 | 10.11.48.1 |

# Objectives

Part 1: Examine the Network Requirements

Part 2: Design the VLSM Addressing Scheme

Part 3: Assign IP Addresses to Devices and Verify Connectivity

# Background

In this activity, you are given a /24 network address to use to design a VLSM addressing scheme. Based on a set of requirements, you will assign subnets and addressing, configure devices and verify connectivity.

# Instructions Part 1: Examine the Network Requirements

Step 1: Determine the number of subnets needed.

You will subnet the network address  	 	. The network has the following requirements:

Packet Tracer - VLSM Design and Implementation Practice

- LAN will require  	 	 host IP addresses

- LAN will require  	 	 host IP addresses

- LAN will require  	 	 host IP addresses

- LAN will require  	 	 host IP addresses

Question:

How many subnets are needed in the network topology?

5

Type your answers here.

Step 2: Determine the subnet mask information for each subnet.

Questions:

- Which subnet mask will accommodate the number of IP addresses required for   	 	?

255.255.255.240/28

How many usable host addresses will this subnet support?

14 hosts

Type your answers here.

- Which subnet mask will accommodate the number of IP addresses required for   	 	?

255.255.255.224/27

How many usable host addresses will this subnet support?

30 hosts

Type your answers here.

- Which subnet mask will accommodate the number of IP addresses required for   	 	?

255.255.255.248/29

How many usable host addresses will this subnet support?

6 hosts

Type your answers here.

- Which subnet mask will accommodate the number of IP addresses required for   	 	?

255.255.255.192/26

How many usable host addresses will this subnet support?

62 hosts

Type your answers here.

- Which subnet mask will accommodate the number of IP addresses required for the connection between   	 	 and  	 	 	?

255.255.255.252/30

Type your answers here.

# Part 2: Design the VLSM Addressing Scheme

Step 1: Divide the  	 	 	 network based on the number of hosts per subnet.

- Use the first subnet to accommodate the largest LAN.

- Use the second subnet to accommodate the second largest LAN.

- Use the third subnet to accommodate the third largest LAN.

- Use the fourth subnet to accommodate the fourth largest LAN.

- Use the fifth subnet to accommodate the connection between  	 	 	 and  	 	.

Step 2: Document the VLSM subnets.

Complete the Subnet Table, listing the subnet descriptions (e.g. [[S1Name]] LAN), number of hosts needed, then network address for the subnet, the first usable host address, and the broadcast address. Repeat until all addresses are listed.

Subnet Table

Packet Tracer - VLSM Design and Implementation Practice

| Subnet Description | Number of Hosts Needed | Network Address/CIDR | First Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| Host-D LAN | 60 | 10.11.48.0/26 | 10.11.48.62 | 10.11.48.63 |
| Host-B LAN | 30 | 10.11.48.64/27 | 10.11.48.94 | 10.11.48.95 |
| Host-A LAN | 14 | 10.11.48.96/28 | 10.11.48.110 | 10.11.48.111 |
| Host-C LAN | 6 | 10.11.48.112/29 | 10.11.48.118 | 10.11.48.119 |
| WAN LINK | 2 | 10.11.48.120/30 | 10.11.48.122 | 10.11.48.123 |

Step 3: Document the addressing scheme.

- Assign the first usable IP addresses to   	 	for the two LAN links and the WAN link.

- Assign the first usable IP addresses to   	 	 for the two LAN links. Assign the last usable IP address for the WAN link.

- Assign the second usable IP addresses to the switches.

- Assign the last usable IP addresses to the hosts.

# Part 3: Assign IP Addresses to Devices and Verify Connectivity

Most of the IP addressing is already configured on this network. Implement the following steps to complete the addressing configuration.

Step 1: Configure IP addressing on the   	 	 router LAN interfaces.

Step 2: Configure IP addressing on the   	 	, switch including the default gateway.

Step 3: Configure IP addressing on  	 	 	, including the default gateway.

Step 4: Verify connectivity.

You can only verify connectivity from  	 	,  	 	, and   	. However, you should be able to ping every IP address listed in the Addressing Table.

End of document