Packet Tracer - Basic Device Configuration

Topology

You will receive one of three possible topologies.

# Addressing Table

| Device | Interface | IP Address | Default Gateway |
| --- | --- | --- | --- |
| Floor14 | G0/0 G0/0 G0/0 | 172.14.5.1/24 | N/A N/A N/A |
|  |  | 2001:DB8:CAFE:1::1/64 |  |
|  | G0/1 G0/1 G0/1 | 172.14.10.1/24 | N/A N/A N/A |
|  |  | 2001:DB8:CAFE:2::1/64 |  |
|  |  | blank |  |
| Room-145 | VLAN 1 | 172.14.5.35/24 | 172.14.5.1 |
| Room-146 | VLAN 1 | 172.14.10.35/24 | 172.14.10.1 |
| Manager-A | NIC NIC | 172.14.5.50/24 | 172.14.5.1 |
|  |  | 2001:DB8:CAFE:1::50/64 | FE80::1 |
| Reception-A | NIC NIC | 172.14.5.60/24 | 172.14.5.1 |
|  |  | 2001:DB8:CAFE:1::60/64 | FE80::1 |
| Manager-B | NIC NIC | 172.14.10.50/24 | 172.14.10.1 |
|  |  | 2001:DB8:CAFE:2::50/64 | FE80::2 |
| Reception-B | NIC NIC | 172.14.10.60/24 | 172.14.10.1 |
|  |  | 2001:DB8:CAFE:2::60/64 | FE80::2 |

Blank Line, No additional information

# Objectives

 	Complete the network documentation.

 	Perform basic device configurations on a router and a switch.

 	Verify connectivity and troubleshoot any issues.

# Scenario

Your network manager is impressed with your performance in your job as a LAN technician. She would like you to demonstrate your ability to configure a router that connects two LANs. Your tasks include configuring basic settings on a router and a switch using the Cisco IOS. You will also configure IPv6 addresses on network devices and hosts. You will then verify the configurations by testing end-to-end connectivity. You goal is to establish connectivity between all devices.

 2013 - 2023 Cisco and/or its affiliates. All rights reserved. Cisco Public 	Page 1 of 2 	www.netacad.com

## Packet Tracer - Basic Device Configuration

Note: The VLAN1 interface on                             will not be reachable over IPv6.

In this activity you will configure the                           router,                             switch, and the                          .

Note: Packet Tracer will not score some configured values, however these values are required to accomplish full connectivity in the network.

# Requirements

|  | Provide the missing information in the Addressing Table. Note: Some of the information is provided in the Packet Tracer instructions for your topology. |
| --- | --- |
|  | Name the router and the second switch . You will not be able to access the switch. |
|  | Use cisco as the user EXEC password for all lines. |
|  | Use class as the encrypted privileged EXEC password. |
|  | Encrypt all plaintext passwords. |
|  | Configure an appropriate banner. |
|  | Configure IPv4 and IPv6 addressing for the switch according to the Addressing Table. |
|  | Configure IPv4 and IPv6 addressing for the switch according to the Addressing Table. |
|  | The hosts are partially configured. Complete the IPv4 addressing, and fully configure the IPv6 addresses according to the Addressing Table. |
|  | Document interfaces with descriptions, including the VLAN 1 interface. |
|  | Save your configurations. |
|  | Verify connectivity between all devices. All devices should be able to ping all other devices with IPv4 and IPv6. |
|  | Troubleshoot and document any issues. |
|  | Implement the solutions necessary to enable and verify full end-to-end connectivity. |

Note: Click Check Results button to see your progress. Click the Reset Activity button to generate a new set of requirements.

End of document

 2013 - 2023 Cisco and/or its affiliates. All rights reserved. Cisco Public 	Page 2 of 2 	www.netacad.com