Packet Tracer - Identify MAC and IP Addresses

# Objectives

Part 1: Gather PDU Information for Local Network Communication

Part 2: Gather PDU Information for Remote Network Communication

# Background

This activity is optimized for viewing PDUs. The devices are already configured. You will gather PDU information in simulation mode and answer a series of questions about the data you collect.

# Instructions Part 1: Gather PDU Information for Local Network Communication

Note: Review the Reflection Questions in Part 3 before proceeding with Part 1. It will give you an idea of the type of information you will need to gather PDU information as a packet travels from 172.16.31.5 to 172.16.31.2.

- Click 172.16.31.5 and open the Command Prompt.

- Enter the ping 172.16.31.2 command.

- Switch to simulation mode and repeat the ping 172.16.31.2 command. A PDU appears next to 172.16.31.5.

- Click the PDU and note the following information from the OSI Model and Outbound PDU Layer tabs:

o Destination MAC Address: 000C:85CC:1DA7 o Source MAC Address: 00D0:D311:C788 o Source IP Address: 172.16.31.5 o Destination IP Address: 172.16.31.2 o At Device: 172.16.31.5

- Click Capture / Forward (the right arrow followed by a vertical bar) to move the PDU to the next device. Gather the same information from Step 1d. Repeat this process until the PDU reaches its destination. Record the PDU information you gathered into a spreadsheet using a format like the table shown below:

Example Spreadsheet Format

| At Device | Dest. MAC | Src MAC | Src IPv4 | Dest IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.5 | 000C:85CC:1DA7 | 00D0:D311:C788 | 172.16.31.5 | 172.16.31.2 |
| Switch1 | 000C:85CC:1DA7 | 00D0:D311:C788 | N/A | N/A |
| Hub | N/A | N/A | N/A | N/A |
| 172.16.31.2 | 00D0:D311:C788 | 000C:85CC:1DA7 | 172.16.31.2 | 172.16.31.5 |

Step 2: Gather additional PDU information from other pings.

Repeat the process in Step 1 and gather the information for the following tests:

  - Ping 172.16.31.2 from 172.16.31.3.

| At Device | Dest. MAC | Src MAC | Src IPv4 | Dest IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.3 | 000C:85CC:1DA7 | 0060.7036.2849 | 172.16.31.3 | 172.16.31.2 |
| Hub | N/A | N/A | 172.16.31.3 | 172.16.31.2 |
| Switch1 | 0060.7036.2849 | 000C:85CC:1DA7 | N/A | N/A |
| 172.16.31.2 | 000C:85CC:1DA7 | 0060.7036.2849 | 172.16.31.2 | 172.16.31.3 |

Cisco and/or its affiliates. All rights reserved. Cisco

Packet Tracer - Identify MAC and IP Addresses

  - Ping 172.16.31.4 from 172.16.31.5.

| At Device | Dest. MAC | Src MAC | Src IPv4 | Dest IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.5 | 000C.CF0B.BC80 | 00D0:D311:C788 | 172.16.31.5 | 172.16.31.4 |
| Switch1 | 000C.CF0B.BC80 | 00D0:D311:C788 | N/A | N/A |
| 172.16.31.4 | 00D0:D311:C788 | 000C.CF0B.BC80 | 172.16.31.4 | 172.16.31.5 |

Return to Realtime mode.

# Part 2: Gather PDU Information for Remote Network Communication

In order to communicate with remote networks, a gateway device is necessary. Study the process that takes place to communicate with devices on the remote network. Pay close attention to the MAC addresses used.

Step 1: Gather PDU information as a packet travels from 172.16.31.5 to 10.10.10.2.

- Click 172.16.31.5 and open the Command Prompt.

- Enter the ping 10.10.10.2 command.

- Switch to simulation mode and repeat the ping 10.10.10.2 command. A PDU appears next to 172.16.31.5.

- Click the PDU and note the following information from the Outbound PDU Layer tab:

  - Destination MAC Address: 00D0:BA8E:741A

  - Source MAC Address: 00D0:D311:C788

  - Source IP Address: 172.16.31.5

  - Destination IP Address: 10.10.10.2

  - At Device: 172.16.31.5

Question:

What device has the destination MAC that is shown?

The router ype your answers here.

- Click Capture / Forward (the right arrow followed by a vertical bar) to move the PDU to the next device. Gather the same information from Step 1d. Repeat this process until the PDU reaches its destination. Record the PDU information you gathered from pinging 172.16.31.5 to 10.10.10.2 into a spreadsheet using a format like the sample table shown below:

| At Device | Dest. MAC | Src MAC | Src IPv4 | Dest IPv4 |
| --- | --- | --- | --- | --- |
| 172.16.31.5 | 00D0:BA8E:741A | 00D0:D311:C788 | 172.16.31.5 | 10.10.10.2 |
| Switch1 | 00D0:BA8E:741A | 00D0:D311:C788 | N/A | N/A |
| Router | 0060:2F84:4AB6 | 00D0:588C:2401 | 172.16.31.5 | 10.10.10.2 |
| Switch0 | 0060:2F84:4AB6 | 00D0:588C:2401 | N/A | N/A |
| Access Point | N/A | N/A | N/A | N/A |
| 10.10.10.2 | 00D0:588C:2401 | 0060:2F84:4AB6 | 10.10.10.2 | 172.16.31.5 |

# Reflection Questions

Answer the following questions regarding the captured data:

- Were there different types of cables/media used to connect devices?

Yes, copper fiber wireless

Type your answers here.

- Did the cables change the handling of the PDU in any way?

No

Type your answers here.

- Did the Hub lose any of the information that it received?

No

Cisco and/or its affiliates. All rights reserved. Cisco

Packet Tracer - Identify MAC and IP Addresses

Type your answers here.

- What does the Hub do with MAC addresses and IP addresses?

The hub did not do anything with the addresses.

Type your answers here.

- Did the wireless Access Point do anything with the information given to it?

Yes, the access point repackaged it as wireless 802.11.

Type your answers here.

- Was any MAC or IP address lost during the wireless transfer?

No

Type your answers here.

- What was the highest OSI layer that the Hub and Access Point used?

Layer 1

Type your answers here.

- Did the Hub or Access Point ever replicate a PDU that was rejected with a red “X”?

Yes

Type your answers here.

- When examining the PDU Details tab, which MAC address appeared first, the source or the destination?

Destination

Type your answers here.

- Why would the MAC addresses appear in this order?

If the destination is listed first a switch can begin forwarding a frame to a known MAC address faster.

- Was there a pattern to the MAC addressing in the simulation?

No

Type your answers here.

- Did the switches ever replicate a PDU that was rejected with a red “X”?

No

Type your answers here.

- Every time that the PDU was sent between the 10 network and the 172 network, there was a point where the MAC addresses suddenly changed. Where did that occur?

It occurred at the Router

Type your answers here.

- Which device uses MAC addresses that start with 00D0:BA?

Router

Type your answers here.

- What devices did the other MAC addresses belong to?

The sender and receiver

Type your answers here.

- Did the sending and receiving IPv4 addresses change fields in any of the PDUs?

No

Type your answers here.

- When you follow the reply to a ping, sometimes called a pong, do you see the sending and receiving IPv4 addresses switch?

Yes

Type your answers here.

- What is the pattern to the IPv4 addressing used in this simulation?

Each port of a router requires a set of unique addresses.

Type your answers here.

- Why do different IP networks need to be assigned to different ports of a router?

The function of the router is to connect different networks so information can be sent between them.

Type your answers here.

- If this simulation was configured with IPv6 instead of IPv4, what would be different?

The IPv4 addresses would be changed to IPv6 addresses but everything else would remain the same.

Type your answers here.

End of document

Cisco and/or its affiliates. All rights reserved. Cisco