Packet Tracer - Examine the ARP Table

# Addressing Table

| Device | Interface | MAC Address | Switch Interface |
| --- | --- | --- | --- |
| Router0 Router0 | Gg0/0 | 0001.6458.2501 | G0/1 |
|  | S0/0/0 | N/A | N/A |
| Router1 Router1 | G0/0 | 00E0.F7B1.8901 | G0/1 |
|  | S0/0/0 | N/A | N/A |
| 10.10.10.2 | Wireless | 0060.2F84.4AB6 | F0/2 |
| 10.10.10.3 | Wireless | 0060.4706.572B | F0/2 |
| 172.16.31.2 | F0 | 000C.85CC.1DA7 | F0/1 |
| 172.16.31.3 | F0 | 0060.7036.2849 | F0/2 |
| 172.16.31.4 | G0 | 0002.1640.8D75 | F0/3 |

# Objectives

Part 1: Examine an ARP Request

Part 2: Examine a Switch MAC Address Table

Part 3: Examine the ARP Process in Remote Communications

# Background

This activity is optimized for viewing PDUs. The devices are already configured. You will gather PDU information in simulation mode and answer a series of questions about the data you collect.

# Instructions Part 1: Examine an ARP Request

Step 1: Generate ARP requests by pinging 172.16.31.3 from 172.16.31.2.

Open a command prompt

- Click 172.16.31.2 and open the Command Prompt.

- Enter the arp -d command to clear the ARP table.

Close a command prompt

- Enter Simulation mode and enter the command ping 172.16.31.3. Two PDUs will be generated. The ping command cannot complete the ICMP packet without knowing the MAC address of the destination. So the computer sends an ARP broadcast frame to find the MAC address of the destination.

- Click Capture/Forward once. The ARP PDU moves Switch1 while the ICMP PDU disappears, waiting for the ARP reply. Open the PDU and record the destination MAC address.

Question:

Is this address listed in the table above?

No its not listed above.

Type your answers here.

Packet Tracer - Examine the ARP Table

- Click Capture/Forward to move the PDU to the next device.

Question:

How many copies of the PDU did Switch1 make?

3 copies

Type your answers here.

What is the IP address of the device that accepted the PDU?

172.16.31.3

Type your answers here.

- Open the PDU and examine Layer 2.

Question:

What happened to the source and destination MAC addresses?

The source address became the destination, FFFF.FFFF.FFFF turned into MAC address of 172.16.31.3

Type your answers here.

- Click Capture/Forward until the PDU returns to 172.16.31.2.

Question:

How many copies of the PDU did the switch make during the ARP reply?

1 copy

Type your answers here.

Step 2: Examine the ARP table.

- Note that the ICMP packet reappears. Open the PDU and examine the MAC addresses.

Question:

Do the MAC addresses of the source and destination align with their IP addresses?

Yes

Type your answers here.

- Switch back to Realtime and the ping completes.

- Click 172.16.31.2 and enter the arp –a command.

Question:

To what IP address does the MAC address entry correspond?

172.16.31.3

Type your answers here.

In general, when does an end device issue an ARP request?

It issues an ARP request when it does not know the receivers MAC address.

Type your answers here.

# Part 2: Examine a Switch MAC Address Table

Step 1: Generate additional traffic to populate the switch MAC address table.

Open a command prompt

- From 172.16.31.2, enter the ping 172.16.31.4 command.

- Click 10.10.10.2 and open the Command Prompt.

- Enter the ping 10.10.10.3 command.

Question:

How many replies were sent and received?

4 replies sent, and 4 received.

Type your answers here.

Close a command prompt

Step 2: Examine the MAC address table on the switches.

- Click Switch1and then the CLI tab. Enter the show mac-address-table command.

Question:

Do the entries correspond to those in the table above?

Yes

Type your answers here.

- Click Switch0, then the CLI tab. Enter the show mac-address-table command.

Questions:

Do the entries correspond to those in the table above?

Yes

Packet Tracer - Examine the ARP Table

Type your answers here.

Why are two MAC addresses associated with one port?

Because both of those devices connect to one port through the access point.

Type your answers here.

# Part 3: Examine the ARP Process in Remote Communications

Step 1: Generate traffic to produce ARP traffic.

Open a command prompt

- Click 172.16.31.2 and open the Command Prompt.

- Enter the ping 10.10.10.1 command.

- Type arp –a.

Question:

What is the IP address of the new ARP table entry?

172.16.31.1

Type your answers here.

- Enter arp -d to clear the ARP table and switch to Simulation mode.

- Repeat the ping to 10.10.10.1.

Question:

How many PDUs appear?

2 PDUs

Type your answers here.

Close a command prompt

- Click Capture/Forward. Click the PDU that is now at Switch1.

Question:

What is the target destination IP destination address of the ARP request?

172.16.31.1

Type your answers here.

- The destination IP address is not 10.10.10.1.

Question:

Why?

Because the receiving device is not on the same network. The local default gateway IP becomes the destination IP. The source device uses the ARP process to find the MAC of the default gateway to complete the ping.ur answers here.

Step 2: Examine the ARP table on Router1.

- Switch to Realtime mode. Click Router1 and then the CLI tab.

- Enter privileged EXEC mode and then the show mac-address-table command.

Question:

How many MAC addresses are in the table? Why?

0, this command means something different, it’s for a layer 2 switch and its asking us to enter the command into a layer 3 router.

Type your answers here.

- Enter the show arp command.

Questions:

Is there an entry for 172.16.31.2?

Yes

Type your answers here.

What happens to the first ping in a situation where the router responds to the ARP request?

It would time out.

Type your answers here.

End of document